# The University of Kansas

School of Engineering

Department of Electrical Engineering and Computer Science

EECS 645 – Computer Architecture

Fall 2015

Homework 02 (Resource Sharing)

Student Name:	Dustin Horvath				Student ID: 2729265

Lecture Example (Template Answer):

Given a resource that is to be shared by two consumers such that only one consumer has access to the resource at any given time. The policy of access is non-preemptive with no priority. More specifically the policy is as follows:

- When the resource is idle and the consumers simultaneously request the resource, their requests are ignored until only one request is submitted

- When the resource is being accessed by any of the consumers, the consumers are scheduled as First-Come-First-Served (FCFS)

Design a two-consumer arbiter/controller that controls the access to the shared resource and implements the above policy. In the design process, provide the following:

- The system architecture (block diagram) showing the interface ports to the arbiter including the clock and reset signals.

- Finite State Machine (FSM) diagram showing all possible states, transitions, and output values.

- K-maps for internal state and output variables.

- Boolean expressions for internal state and output variables.

- Detailed logic diagram using synchronous memory elements showing internal connections and external interfaces.

- Complete description of the arbiter using VHDL code.

- Complete VHDL testbench along with simulation results.

Figure 1: System architecture

Table 1: Input Code Assignment

| Input Combinations (Input Codes) |  | Input Description |
| --- | --- | --- |
| REQ_01 | REQ_02 |  |
| 0 | 0 | No requests |
| 0 | 1 | Consumer 2 requests resource |
| 1 | 0 | Consumer 1 requests resource |
| 1 | 1 | Both consumers request resource |

Table 2: Output Code Assignment

| Output Combinations (Output Codes) |  | Output Description |
| --- | --- | --- |
| ACK_01 | ACK_02 |  |
| 0 | 0 | None granted access to resource |
| 0 | 1 | Consumer 2 granted access to resource |
| 1 | 0 | Consumer 1 granted access to resource |
| 1 | 1 | Forbidden output |

Table 3: State Code Assignment

| State Description |  | State Codes |  |
| --- | --- | --- | --- |
|  |  | S_01 | S_02 |
| Required States | Resource is idle (No access) | 0 | 0 |
|  | Resource is used by consumer 2 (Con_02) | 0 | 1 |
|  | Resource is used by consumer 1 (Con_01) | 1 | 0 |
| Unused States | Extra unused state (Unused) | 1 | 1 |

Figure 2: State transition diagram

Table 4: State transition table

| Next State (S1 , S2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 | 00 | 01 | 00 | 10 |
|  | 01 | 00 | 01 | 01 | 00 |
|  | 11 | 00 | 00 | 00 | 00 |
|  | 10 | 00 | 00 | 10 | 10 |

Table 5: K-map for the state variable S1

| Next State (S1) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  |  |  | 1 |
|  | 01 |  |  |  |  |
|  | 11 |  |  |  |  |
|  | 10 |  |  | 1 | 1 |

Table 6: K-map for the state variable S2

| Next State (S2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  | 1 |  |  |
|  | 01 |  | 1 | 1 |  |
|  | 11 |  |  |  |  |
|  | 10 |  |  |  |  |

Table 7: Output transition table

| Outputs (ACK1 , ACK2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 | 00 | 00 | 00 | 00 |
|  | 01 | 01 | 01 | 01 | 01 |
|  | 11 | 00 | 00 | 00 | 00 |
|  | 10 | 10 | 10 | 10 | 10 |

Table 8: K-map for the output variable ACK1

| Outputs (ACK1) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  |  |  |  |
|  | 01 |  |  |  |  |
|  | 11 |  |  |  |  |
|  | 10 | 1 | 1 | 1 | 1 |

Table 9: K-map for the output variable ACK2

| Outputs (ACK2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  |  |  |  |
|  | 01 | 1 | 1 | 1 | 1 |
|  | 11 |  |  |  |  |
|  | 10 |  |  |  |  |

Figure 3: Synchronous logic diagram

Structural VHDL code

Testbench and Simulation Results

Behavioral VHDL code

Testbench and Simulation Results

Homework Problem I:

Given a resource that is to be shared by two consumers such that only one consumer has access to the resource at any given time. The policy of access is non-preemptive with priority determined by the consumer index. More specifically the policy is as follows:

- When the resource is idle and the consumers simultaneously request the resource, the consumers are prioritized according to their consumer number (i.e. consumer 2 has a higher priority than consumer 1)

- When the resource is being accessed by any of the consumers, the consumers are scheduled as First-Come-First-Served (FCFS)

Design a two-consumer arbiter/controller that controls the access to the shared resource and implements the above policy. In the design process, provide the following:

- The system architecture (block diagram) showing the interface ports to the arbiter including the clock and reset signals.

- Finite State Machine (FSM) diagram showing all possible states, transitions, and output values.

- K-maps for internal state and output variables.

- Boolean expressions for internal state and output variables.

- Detailed logic diagram using synchronous memory elements showing internal connections and external interfaces.

- Complete description of the arbiter using VHDL code.

- Complete VHDL testbench along with simulation results.

Figure 1: System architecture

Table 1: Input Code Assignment

| Input Combinations (Input Codes) |  | Input Description |
| --- | --- | --- |
| REQ_01 | REQ_02 |  |
| 0 | 0 | No Requests |
| 0 | 1 | Consumer 2 requests resource |
| 1 | 0 | Consumer 1 requests resource |
| 1 | 1 | Both consumers request resource |

Table 2: Output Code Assignment

| Output Combinations (Output Codes) |  | Output Description |
| --- | --- | --- |
| ACK_01 | ACK_02 |  |
| 0 | 0 | None granted access to resource |
| 0 | 1 | Consumer 2 granted access to resource |
| 1 | 0 | Consumer 1 granted access to resource |
| 1 | 1 | Forbidden output |

Table 3: State Code Assignment

| State Description |  | State Codes |  |
| --- | --- | --- | --- |
|  |  | S_01 | S_02 |
| Required States | Resource is idle (No access) | 0 | 0 |
|  | Resource is used by consumer 2 (Con_02) | 0 | 1 |
|  | Resource is used by consumer 1 (Con_01) | 1 | 0 |
| Unused States | Extra unused state (Unused) | 1 | 1 |

Figure 2: State transition diagram

Table 4: State transition table

| Next State (S1 , S2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 | 00 | 01 | 01 | 10 |
|  | 01 | 00 | 01 | 01 | 00 |
|  | 11 | 00 | 00 | 00 | 00 |
|  | 10 | 00 | 00 | 10 | 10 |

Table 5: K-map for the state variable S1

| Next State (S1 ) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  |  |  | 1 |
|  | 01 |  |  |  |  |
|  | 11 |  |  |  |  |
|  | 10 |  |  | 1 | 1 |

Table 6: K-map for the state variable S2

| Next State (S2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  | 1 | 1 |  |
|  | 01 |  | 1 | 1 |  |
|  | 11 |  |  |  |  |
|  | 10 |  |  |  |  |

Table 7: Output transition table

| Outputs (ACK1 , ACK2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 | 00 | 00 | 00 | 00 |
|  | 01 | 01 | 01 | 01 | 01 |
|  | 11 | 00 | 00 | 00 | 00 |
|  | 10 | 10 | 10 | 10 | 10 |

Table 8: K-map for the output variable ACK1

| Outputs (ACK1) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  |  |  |  |
|  | 01 |  |  |  |  |
|  | 11 |  |  |  |  |
|  | 10 | 1 | 1 | 1 | 1 |

Table 9: K-map for the output variable ACK2

| Outputs (ACK2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  |  |  |  |
|  | 01 | 1 | 1 | 1 | 1 |
|  | 11 |  |  |  |  |
|  | 10 |  |  |  |  |

Figure 3: Synchronous logic diagram

Structural VHDL code

Testbench and Simulation Results

Behavioral VHDL code

Testbench and Simulation Results

Homework Problem II:

Given a resource that is to be shared by two consumers such that only one consumer has access to the resource at any given time. The policy of access is non-preemptive with priority determined by the consumer index. More specifically the policy is as follows:

- When the resource is idle and the consumers simultaneously request the resource, the consumers are prioritized according to their consumer number (i.e. consumer 1 has a higher priority than consumer 2)

- When the resource is being accessed by any of the consumers, the consumers are scheduled as First-Come-First-Served (FCFS)

Design a two-consumer arbiter/controller that controls the access to the shared resource and implements the above policy. In the design process, provide the following:

- The system architecture (block diagram) showing the interface ports to the arbiter including the clock and reset signals.

- Finite State Machine (FSM) diagram showing all possible states, transitions, and output values.

- K-maps for internal state and output variables.

- Boolean expressions for internal state and output variables.

- Detailed logic diagram using synchronous memory elements showing internal connections and external interfaces.

- Complete description of the arbiter using VHDL code.

- Complete VHDL testbench along with simulation results.

Figure 1: System architecture

Table 1: Input Code Assignment

| Input Combinations (Input Codes) |  | Input Description |
| --- | --- | --- |
| REQ_01 | REQ_02 |  |
| 0 | 0 | No Requests |
| 0 | 1 | Consumer 2 requests resource |
| 1 | 0 | Consumer 1 requests resource |
| 1 | 1 | Both consumers request resource |

Table 2: Output Code Assignment

| Output Combinations (Output Codes) |  | Output Description |
| --- | --- | --- |
| ACK_01 | ACK_02 |  |
| 0 | 0 | None granted access to resource |
| 0 | 1 | Consumer 2 granted access to resource |
| 1 | 0 | Consumer 1 granted access to resource |
| 1 | 1 | Forbidden output |

Table 3: State Code Assignment

| State Description |  | State Codes |  |
| --- | --- | --- | --- |
|  |  | S_01 | S_02 |
| Required States | Resource is idle (No access) | 0 | 0 |
|  | Resource is used by consumer 2 (Con_02) | 0 | 1 |
|  | Resource is used by consumer 1 (Con_01) | 1 | 0 |
| Unused States | Extra unused state (Unused) | 1 | 1 |

Figure 2: State transition diagram

Table 4: State transition table

| Next State (S1 , S2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 | 00 | 01 | 10 | 10 |
|  | 01 | 00 | 01 | 01 | 00 |
|  | 11 | 00 | 00 | 00 | 00 |
|  | 10 | 00 | 00 | 10 | 10 |

Table 5: K-map for the state variable S1

| Next State (S1 ) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  |  | 1 | 1 |
|  | 01 |  |  |  |  |
|  | 11 |  |  |  |  |
|  | 10 |  |  | 1 | 1 |

Table 6: K-map for the state variable S2

| Next State (S2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  | 1 |  |  |
|  | 01 |  | 1 | 1 |  |
|  | 11 |  |  |  |  |
|  | 10 |  |  |  |  |

Table 7: Output transition table

| Outputs (ACK1 , ACK2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 | 00 | 00 | 00 | 00 |
|  | 01 | 01 | 01 | 01 | 01 |
|  | 11 | 11 | 11 | 11 | 11 |
|  | 10 | 10 | 10 | 10 | 10 |

Table 8: K-map for the output variable ACK1

| Outputs (ACK1) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  |  |  |  |
|  | 01 |  |  |  |  |
|  | 11 |  |  |  |  |
|  | 10 | 1 | 1 | 1 | 1 |

Table 9: K-map for the output variable ACK2

| Outputs (ACK2) |  | Inputs (REQ1 , REQ2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| Current State (S1 , S2) | 00 |  |  |  |  |
|  | 01 | 1 | 1 | 1 | 1 |
|  | 11 |  |  |  |  |
|  | 10 |  |  |  |  |

Figure 3: Synchronous logic diagram

Structural VHDL code

Testbench and Simulation Results

Behavioral VHDL code

Testbench and Simulation Results

Homework Problem III:

Given a resource that is to be shared by three consumers such that only one consumer has access to the resource at any given time. The policy of access is non-preemptive with no priority. More specifically the policy is as follows:

- When the resource is idle and the consumers simultaneously request the resource, their requests are ignored until only one request is submitted

- When the resource is being accessed by any of the consumers, the consumers are scheduled as First-Come-First-Served (FCFS)

Design a three-consumer arbiter/controller that controls the access to the shared resource and implements the above policy. In the design process, provide the following:

- The system architecture (block diagram) showing the interface ports to the arbiter including the clock and reset signals.

- Finite State Machine (FSM) diagram showing all possible states, transitions, and output values.

- K-maps for internal state and output variables.

- Boolean expressions for internal state and output variables.

- Detailed logic diagram using synchronous memory elements showing internal connections and external interfaces.

- Complete description of the arbiter using VHDL code.

- Complete VHDL testbench along with simulation results.

Figure 1: System architecture

Table 1: Input Code Assignment

| Input Combinations (Input Codes) |  |  | Input Description |
| --- | --- | --- | --- |
| REQ_01 | REQ_02 | REQ_03 |  |
| 0 | 0 | 0 | No Requests |
| 0 | 0 | 1 | Consumer 3 requests resource |
| 0 | 1 | 1 | Consumers 2 and 3 requesting |
| 0 | 1 | 0 | Consumer 2 Requests resource |
| 1 | 1 | 0 | Consumers 1 and 2 requesting |
| 1 | 1 | 1 | Consumers 1, 2, and 3 requesting |
| 1 | 0 | 1 | Consumers 1 and 3 requesting |
| 1 | 0 | 0 | Consumer 1 requests resource |

Table 2: Output Code Assignment

| Output Combinations (Output Codes) |  |  | Output Description |
| --- | --- | --- | --- |
| ACK_01 | ACK_02 | ACK_03 |  |
| 0 | 0 | 0 | None granted access to resource |
| 0 | 0 | 1 | Consumer 3 granted access to resource |
| 0 | 1 | 1 | Forbidden output |
| 0 | 1 | 0 | Consumer 2 granted access to resource |
| 1 | 1 | 0 | Forbidden output |
| 1 | 1 | 1 | Forbidden output |
| 1 | 0 | 1 | Forbidden output |
| 1 | 0 | 0 | Consumer 1 granted access to resource |

Table 3: State Code Assignment

| State Description | State Codes |  |
| --- | --- | --- |
|  | S_01 | S_02 |
| Resource is idle | 0 | 0 |
| Resource is used by consumer 3 | 0 | 1 |
| Resource is used by consumer 1 | 1 | 0 |
| Resource is used by consumer 2 | 1 | 1 |

Figure 2: State transition diagram

Table 4: State transition table

| Next State (S1 , S2) |  | Inputs (REQ1 , REQ2 , REQ3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 000 | 001 | 011 | 010 | 110 | 111 | 101 | 100 |
| Current State (S1 , S2) | 00 | 00 | 01 | 00 | 11 | 00 | 00 | 00 | 10 |
|  | 01 | 00 | 01 | 01 | 00 | 00 | 01 | 01 | 00 |
|  | 11 | 00 | 00 | 11 | 11 | 11 | 11 | 00 | 00 |
|  | 10 | 00 | 00 | 00 | 00 | 10 | 10 | 10 | 10 |

Table 5: K-map for the state variable S1

| Next State (S1) |  | Inputs (REQ1 , REQ2 , REQ3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 000 | 001 | 011 | 010 | 110 | 111 | 101 | 100 |
| Current State (S1 , S2) | 00 |  |  |  | 1 |  |  |  | 1 |
|  | 01 |  |  |  |  |  |  |  |  |
|  | 11 |  |  | 1 | 1 | 1 | 1 |  |  |
|  | 10 |  |  |  |  | 1 | 1 | 1 | 1 |

Table 6: K-map for the state variable S2

| Next State (S2) |  | Inputs (REQ1 , REQ2 , REQ3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 000 | 001 | 011 | 010 | 110 | 111 | 101 | 100 |
| Current State (S1 , S2) | 00 |  | 1 |  | 1 |  |  |  |  |
|  | 01 |  | 1 | 1 |  |  | 1 | 1 |  |
|  | 11 |  |  | 1 | 1 | 1 | 1 |  |  |
|  | 10 |  |  |  |  |  |  |  |  |

Table 7: Output transition table

| Outputs (ACK1 , ACK2 , ACK3) |  | Inputs (REQ1 , REQ2 , REQ3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 000 | 001 | 011 | 010 | 110 | 111 | 101 | 100 |
| Current State (S1 , S2) | 00 | 000 | 000 | 000 | 000 | 000 | 000 | 000 | 000 |
|  | 01 | 001 | 001 | 001 | 001 | 001 | 001 | 001 | 001 |
|  | 11 | 010 | 010 | 010 | 010 | 010 | 010 | 010 | 010 |
|  | 10 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |

Table 8: K-map for the output variable ACK1

| Outputs (ACK1) |  | Inputs (REQ1 , REQ2 , REQ3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 000 | 001 | 011 | 010 | 110 | 111 | 101 | 100 |
| Current State (S1 , S2) | 00 |  |  |  |  |  |  |  |  |
|  | 01 |  |  |  |  |  |  |  |  |
|  | 11 |  |  |  |  |  |  |  |  |
|  | 10 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Table 9: K-map for the output variable ACK2

| Outputs (ACK2) |  | Inputs (REQ1 , REQ2 , REQ3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 000 | 001 | 011 | 010 | 110 | 111 | 101 | 100 |
| Current State (S1 , S2) | 00 |  |  |  |  |  |  |  |  |
|  | 01 |  |  |  |  |  |  |  |  |
|  | 11 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | 10 |  |  |  |  |  |  |  |  |

Table 10: K-map for the output variable ACK3

| Outputs (ACK3) |  | Inputs (REQ1 , REQ2 , REQ3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 000 | 001 | 011 | 010 | 110 | 111 | 101 | 100 |
| Current State (S1 , S2) | 00 |  |  |  |  |  |  |  |  |
|  | 01 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | 11 |  |  |  |  |  |  |  |  |
|  | 10 |  |  |  |  |  |  |  |  |

Figure 3: Synchronous logic diagram

Structural VHDL code

Testbench and Simulation Results

Behavioral VHDL code

Testbench and Simulation Results